Hardware Implemented Real-Time Edge-Based Tomato Leaf Disease Classification using YOLOv11n

Shubham Kumar
Department of Electronics and Communication
National Institute of Technology Patna, Bihar, India
shubhamk.pg24.ec@nitp.ac.in

Bharat Gupta
Department of Electronics and Communication
National Institute of Technology Patna, Bihar India
bharat@nitp.ac.in

Shanti Jaiswal
Department of Electronics and Communication
National Institute of Technology Patna, Bihar India
shantij.ph23.ec@nitp.ac.in

Abstract—Tomato leaf diseases pose a significant challenge to crop yield and severely affect tomato production. To mitigate it, timely detection is crucial. This paper presents a real-time edge-based system for tomato leaf disease classification using the lightweight YOLOv11 nano model, designed for deployment on resource constrained devices. The proposed approach performs on-device inference, reducing latency and minimizing reliance on continuous network connectivity. To support large-scale field monitoring, the system utilizes LoRa communication for transmitting inference results tagged with GPS coordinates from distributed edge nodes to a central base station. Then the collected data is uploaded to the Firebase cloud database and visualized through a web based interface which incorporates a grid based field visualization to spatially map disease distribution and provides real time updates to users, along with periodic notifications via Telegram. The model is trained on a dataset comprising over 13,000 tomato leaf images across seven distinct classes (six diseases and one healthy), achieved 95.8% classification accuracy and fast inference speed of 2.7 ms per image. The system was deployed and evaluated on edge hardware in a real world environment, achieving a classification accuracy of 90.12% over 5.15 frame per second.

Keywords—Computer Vision, YOLO, Image Classification, Plant Disease Classification

- INTRODUCTION

Tomato is one of the most widely cultivated crops and plays a significant role in global agriculture due to its nutritional value and economic importance. In India, tomato cultivation is a cornerstone of agricultural productivity, accounting for approximately 11% of the country's total vegetable output. For specialized farmers, it serves as a primary economic driver, contributing as much as 32.2% to their total annual gross household income [1]. However, tomato yield is often threatened by various leaf diseases such as early blight, late blight, bacterial leaf spot, leaf mold, septoria leaf spot and yellow leaf curl virus. These diseases can spread rapidly under favorable environmental conditions and lead to severe yield losses if not detected and treated at an early stage. Traditionally, disease detection relies on manual inspection by farmers or experts, which is time-consuming, subjective and prone to errors. The process becomes even more challenging in real-world field conditions due to large farm lands, irregular or delayed inspection. Delayed or inaccurate diagnosis often results in ineffective treatment and significant economic losses [2]. Recent advancements in deep learning, particularly models from the YOLO family, have shown promising results in automated plant disease detection and classification [3]. But most existing approaches are cloud dependent and require continuous internet connectivity, limiting their deployment in rural and resource constrained environments. Furthermore, many solutions focus primarily on model accuracy without addressing real time deployment, communication and field level monitoring.

To address these challenges, this paper proposes a real time edge based system for tomato leaf disease classification using the lightweight YOLOv11n model. The system performs on-device inference and integrates LoRa communication to transmit classification results along with GPS coordinates to a central base station. The data is stored in the Firebase cloud database and visualized through a web-based interface, which provides real-time updates and periodic notifications via Telegram. A grid based visualization is incorporated to map disease distribution spatially across the field. The proposed system is trained on a dataset of over 13,000 images across six disease classes and one healthy class and is validated in real world conditions, demonstrating its effectiveness for scalable precision agriculture applications.

Figure 1. Grid-based field visualization for disease mapping.

Figure 1 represents a grid-based agricultural field visualization, where the entire field is divided into numbered cells for structured monitoring. Each grid cell corresponds to a specific region, enabling localized analysis of crop conditions and disease distribution. This representation facilitates efficient tracking, comparison and decision making, allowing users to quickly identify affected zones and take targeted actions for precision agriculture. The remaining part of this paper is organized as follows. Section II represents a comprehensive literature review of existing approaches. Section III focus on the system modelling. Section IV represents methodology of proposed method. Section V presents performance evaluation and  comparative analysis of the proposed approach. Finally, Section VI provides the conclusion and future scope.

- RELATED WORK

Researchers have investigated various machine learning and deep learning techniques with the intent of using an early and precise identification of plant leaf diseases to enhance agricultural production by increasing the effectiveness of identifying diseased plants. In the past, most of the methods for identifying diseased plants were based on developing handcrafted features coupled with classical classifiers, but most encountered issues due to variations in the lighting, complexity of the background and disease pattern. However, with deep learning techniques such as CNNs and Object Detection Models, these challenges have been significantly reduced in recognition processing for plant diseases [4]. The current emphasis in scientific research has been focused on utilizing object detection frameworks with a particular interest in utilizing object detection models developed by the YOLO (You Only Look Once) family of models as they are capable of detecting objects and providing real-time detection of these objects with a high level of accuracy. The group of researchers conducted an analysis on each model's ability to accurately detect tomato leaf disease, particularly focusing on the analysis of each YOLO from the YOLOv8, YOLOv9, YOLOv10 and YOLOv11 series. The collective findings indicate that the YOLOv11 model produced the best relative accuracy and efficiency attributes and therefore may be ideally suited for use within the field of precision agricultural applications [5]. Improved versions of YOLO have been suggested by various researchers for more precise disease detection. One example is an improved version of the YOLOv8n algorithm, which uses enhanced feature extraction and increased robustness across different environmental conditions to successfully identify tomato diseases. Another example is the enhanced YOLOv10n algorithm, which incorporates multi-scale feature fusion as well as optimized detection heads, both have demonstrated that architectural changes can lead to serious improvements in both the classification and recall accuracy of disease detection [6].

Apart from YOLO-style methods, various non-YOLO style architectures have also been examined. For example, Transformers & CNNs have been combined into a single model like TomFormer, which provides competitive accuracy when detecting tomato disease by capturing local and global information from both types of networks. Hybrid models, such as Capsule-YOLO, have been proposed to improve how features are represented (segmenting) under complicated field conditions by being robust to occlusions and overlaps of leaves while achieving good accuracy. Most existing research concentrates primarily on enhancing the accuracy of their networks using controlled datasets like PlantVillage, which provides no realistic representation of actual growing conditions. Most of the methods required for deployment will utilize a cloud server for computing so that they must have an active internet connection and require considerable computing power to perform well enough during use. Therefore, implementing these applications in rural farm settings or in large area operations will be challenging, as either the coverage or reliability of the internet network found outside of great metropolitan cities will be very poor and therefore would create long latency issues or service disruption [7].

The DM-YOLO method addresses some of the issues that can occur during real world deployments by introducing alternative based approaches to improving the overall quality of the detection process using factors such as environment variation due to ambient lighting, occlusions and to deal with small lesions present on the plant. For instance, by developing lossy and dynamic methods to employ up-sampling, the DM-YOLO model has been developed to improve the ability of detecting tomato diseases in an overlapping manner under the extreme conditions of fields where diseases are often commonly found. Research into the development of an automated pest/disease detection system in a real-time manner that can be implemented, as opposed to establishing one but not yet usable, has also been the focus of YOLO-based researchers [8]. A fundamental drawback in present methodologies is that nearly all do not integrate a comprehensive end-to-end system involving elements such as edge computing, communications and visualization at the user level. Also, very few studies have included the use of low power communication technologies such as LoRa or provided insights at a spatial field level to aid in decision making.

The current proposed research effort will focus not only on accurately classifying diseases through the use of a lightweight YOLOv11n model, but also take into consideration issues related to deployment, including edge based inferences, long range LoRa communications, cloud based integration and grid-based field visualization. Using this holistic method will eliminate the existing disconnect between developing models and implementing those models in agriculture, thereby creating scalable and efficient solutions to precision farming.

- SYSTEM  MODELLING

The proposed system is designed as an end-to-end framework for real-time tomato leaf disease monitoring, integrating edge computing, long-range communication, cloud storage and visualization. The architecture consists of distributed edge nodes deployed across the agricultural field, a central base station, where each edge node captures leaf images and performs on-device inference using the lightweight YOLOv11n classification model. The inference results, along with associated metadata such as GPS and grid cell coordinates, are transmitted to the base station using LoRa communication. The base station forwards the aggregated data to a cloud platform for storage and visualization.

Figure 2. Proposed system model.

Figure 2 represents the proposed system which consists of a set of distributed edge nodes, a base station, a cloud server and a user interface. Let the set of edge nodes be represented in  equation 1.

E=( E1,     E2,   ….   En )                                     (1)

where each Ei represents an individual edge node deployed in the agricultural field. The system also includes a base station B, a cloud server C and a user interface U for visualization and monitoring. Each edge node captures an image of the crop and performs disease classification locally using the trained YOLOv11n model. The inference process is defined in equation 2.

yi=f(Ii; θ)                                                           (2)

where Ii denotes the input image captured by node Ei, f(.) represents the trained classification model and θ denotes the model parameters. The output yi corresponds to one of the predefined classes (six disease classes or one healthy class). Instead of transmitting raw images, each edge node sends only the inference result along with metadata using LoRa communication. The transmitted data packet is defined in equation 3.

Pi={ IDi​,yi​,(lati​,loni​), ti }             (3)​

where IDi is the node identifier, yi is the predicted class, (lati, loni) represents the GPS coordinates and ti denotes the timestamp. The collected data packets from all nodes are aggregated at the base station and forwarded to the cloud, which is represented in equation 4.

D=g(P1,    P2​,...,   Pn​)                         (4)

where g(.) denotes the aggregation function. To enable spatial visualization, the agricultural field is divided into a grid structure. Each grid cell is mapped to a disease class based on the cell and GPS coordinates.

- THE  METHODOLOGY

The proposed methodology presents an end-to-end framework for real-time tomato leaf disease classification, combining deep learning-based image analysis with edge deployment and efficient data transmission. The system leverages the lightweight YOLOv11n model for on-device inference, enabling fast and accurate classification of leaf conditions directly at the edge node. The overall methodology consists of four major stages.

- Data acquisition and pre-processing

- Model training and edge optimization

- Inference at the edge

- Transmission and reception of results for visualization

- By performing inference locally and transmitting only the predicted labels along with metadata, the system minimizes communication overhead and ensures scalability for large agricultural deployments. Ultimately, this framework offers a robust and resource-efficient solution for precision agriculture. By balancing high-accuracy deep learning with edge computing, the system bridges the gap between sophisticated diagnostic tools and practical, real-time field application.

Figure 3. Block diagram of  proposed methodology

Figure 3 represents the the block diagram of proposed methodology where the dataset used in this study comprises tomato leaf images categorized into six disease classes and one healthy class (refer Table 1), namely, early blight, late blight, bacterial leaf spot, leaf mold, septoria leaf spot, yellow leaf curl virus and healthy leaves taken from online available dataset (plant village and plant doc). The dataset contains 13,657 images, with a relatively balanced distribution across all classes, ensuring unbiased learning. The dataset is divided into training and validation sets, consisting of 10,923 and 2,734 images, respectively. This split allows effective evaluation of the model’s generalization capability. Prior to training, the dataset undergoes pre-processing to enhance model robustness and performance. The pre-processing steps include image re-sizing to match the input requirements of YOLOv11n, normalization of pixel values and data augmentation techniques such as flipping, rotation and scaling. These techniques help the model generalize better under varying environmental conditions such as lighting variations and background complexity. The model is trained for 40 epochs using the ultralytics framework. The training process minimizes classification loss using back-propagation with the AdamW optimizer. The dataset split ensures proper validation, preventing over-fitting and improving generalization. During deployment, the trained model performs inference on edge devices. The predicted class label is combined with GPS coordinates and transmitted via LoRa communication for further processing and visualization.

Table 1. Class names and their instances in the final dataset

| Classname | Instances/Images |
| --- | --- |
| Tomato Early Blight | 2070 |
| Tomato Bacterial Leaf Spot | 1789 |
| Tomato Septoria Leaf Spot | 1849 |
| Tomato Leaf Mold | 2007 |
| Tomato Yellow Leaf Curl Virus | 2035 |
| Tomato Late Blight | 1981 |
| Tomato Healthy | 1926 |

Table 1 breakdown the each class instances present in the final dataset which is used for the training and validation.

The proposed system is implemented using a distributed edge-based architecture consisting of multiple sensing nodes, a base station and a cloud interface. Each edge node is deployed in the agricultural field to capture leaf images and perform real-time disease classification. The base station is responsible for receiving data from edge nodes and forwarding it to the cloud for storage and visualization. Each edge node is built using a Raspberry Pi 4, integrated with a RPi camera v2 module for image acquisition, a NEO-6M GPS module for location tagging and a SX1278 LoRa transceiver for communication. The trained YOLOv11n model is deployed on the edge device to perform real-time disease classification. The AES 128bit encrypted inference results, along with GPS coordinates, are transmitted via LoRa to a base station equipped with a NodeMCU, which forwards the data to the Firebase cloud database. The stored data is then visualized through a web interface, providing real-time monitoring and periodic updates to users via Telegram. Details of components which are used for hardware implementation is mention in Table 2.

Table 2. Implementation hardware

| Hardware Name | Specification |
| --- | --- |
| Raspberry Pi 4 Model B | 4GB RAM, Cortex-A72 64-bit SoC, Broadcom BCM2711 |
| Raspberry Pi Camera Module v2 | 8-megapixel, Hz FoV 62.2o and Vz FoV 48.8o |
| LoRa SX1278 | Semtech SX1278 chip, 433 MHz, sensitivity -148 dBm |
| GPS NEO-6M | Accuracy 2.5 meters, Update rate 5 Hz |
| NodeMCU | ESP8266, ESP-12E , clock speed 80 MHz, Integrated IEEE 802.11 b/g/n Wi-Fi |

- RESULT AND DISCUSSION

The performance of the proposed YOLOv11n classification model is evaluated on the validation dataset comprising 2,734 images across seven classes. The model achieves a Top-1 accuracy of 95.83% and Top-5 accuracy of 99.74%, demonstrating strong classification capability across both diseased and healthy leaf categories.

Figure 4. Learning curve of  proposed methodology

Figure 4 illustrates the training and validation performance of the proposed YOLOv11n model over 40 epochs. The training and validation loss curves show a consistent decreasing trend, indicating effective learning and convergence of the model. The Top-1 accuracy curve shows a sharp increase in the early stages of training, followed by gradual improvement and stabilization around 95.83%, indicating strong classification performance. The close alignment between training and validation loss curves suggests minimal over-fitting, confirming good generalization capability of the model on unseen data.

Figure 5. Confusion matrix of  proposed methodology

Figure 5 represents the confusion matrix of the proposed model, illustrating class-wise prediction performance. The model demonstrates high true positive rates across most classes. Minor misclassifications are observed between visually similar disease classes, which is expected due to overlapping features. Finally the model is converged to classify disease distinctively.

| Classifier name | Dataset images | Accuracy | Inference Time | Trainable Parameters |
| --- | --- | --- | --- | --- |
| ANN [9] | 8,000 | 92.17% | 105 ms | 1 million |
| SVM [9] | 2,000 | 84.23% | 100 ms | Non-parametric |
| RF [9] | 4000 | 79.67% | 102 ms | Non-parametric |
| VGG16 [10] | 12,000 | 92.24% | 22 ms | 138.3 million |
| ResNet50 [11] | 28,000 | 88.38% | 6.5 ms | 25.6 million |
| Custom CNN [12] | 54000 | 92.60% | 12.6 ms | 5 million |
| AlexNet [13] | 14000 | 86.80% | 16.6 ms | 60 million |
| MobileNetV2 + SVM [14] | 380 | 94.53% | 23.57 ms | 3.5 million |
| YOLOv8n [15] | 13000 | 92.72% | 1.80 ms | 3.15 million |
| YOLOv11n (Proposed) | 13657 | 95.83% | 2.70 ms | 1.53 million |

- Table 3. Comparative study

- Table 3 represents a comparative analysis of the proposed YOLOv11n model with traditional machine learning classifiers and recent deep learning approaches. The proposed model achieves the highest accuracy of 95.8%, outperforming others. While YOLOv8n demonstrates slightly lower inference time (1.8 ms), the proposed model maintains competitive speed (2.7 ms) with significantly improved accuracy. Furthermore, the model remains lightweight, with approximately 1.53 million parameters and a computational complexity of 3.2 GFLOPs, making it suitable for real-time and resource-constrained applications.

Figure 6.   Hardware implementation of the proposed methodology

Leaf Images

Rx Antenna

Tx Antenna

Rx status LED

LoRa Tx

Raspberry Pi4

LoRa Rx

GPS NEO 6M

Tx status LED

Pi Camera

Node MCU

- Figure 6 illustrates the hardware workflow of the proposed system. First, a Pi camera captures live video frames, which are processed by a Raspberry Pi to generate inferences. These results are then secured using AES 128-bit encryption and transmitted via LoRa. On the receiving end, a LoRa receiver captures the incoming data, which is then decrypted by a NodeMCU, stored locally and uploaded to a cloud database.

- CONCLUSION

- This study presents a comparative analysis and hardware implementation of the YOLO11 nano model for tomato leaf disease classification. Capable of identifying seven distinct classes—six diseases and one healthy—the model achieved an impressive 95.8% classification accuracy with a rapid inference speed of 2.7 ms per image. When evaluated on edge hardware in a real-world environment, the system maintained a classification accuracy of 90.12% at 5.15 frames per second, consistently outperforming alternative approaches. These results demonstrate that the YOLO11 nano model offers an optimal balance of stability, efficiency and reliability for edge deployment. Future research will focus on integrating a lightweight backbone to further enhance both inference speed and accuracy..

- REFERENCES

- “Economic Analysis of Tomato Cultivation in Himachal Pradesh" is: H. Sharma, S. Sood and V. Kumar, Indian J. Econ. Dev., vol. 17, no. 4, pp. 960-965, Dec. 2021, doi: 10.35716/IJED/20213.

- A. Chelladurai, D. P. Manoj Kumar, S. S. Askar and M. Abouhawwash, “Classification of tomato leaf disease using transductive long short-term memory with an attention mechanism,” Front. Plant Sci., vol. 15, Jan. 2024, doi: 10.3389/fpls.2024.1467811.

- N. Duhan, R. Sujatha and J. S. Jayasankar, “Tomato plant disease detection using YOLO11,” in Proc. INDIACom 2025, New Delhi, India, 2025.

- A. S. Paymode and V. B. Malode, "Efficient Deep Learning Models for Plant Disease Detection: A Systematic Review," arXiv preprint arXiv:2308.14087, 2023. [Online]. Available: https://arxiv.org/abs/2308.14087

- M. K. Tripathi et al., "Techno-economic assessment of tomato cultivation under different protected structures," Sci. Rep., vol. 15, no. 1, Art. no. 11064, Feb. 2025. [Online]. Available: https://www.nature.com/articles/s41598-025-11064-0

- M. J. M. Alhariri, "Analysis of Global and Local Tomato Production: Growth Trends and Challenges," Int. J. Biol. Life Sci., vol. 3, no. 3, pp. 206–211, Dec. 2024. [Online]. Available: https://drpress.org/ojs/index.php/ijbls/article/view/27488

- S. V. Deshpande, A. S. Patil and R. M. Kulkarni, "Agricultural Productivity Forecasting using Deep Learning Models: A Case Study on Tomato Crops in India," arXiv preprint arXiv:2507.03219, Jul. 2025. [Online]. Available: https://arxiv.org/abs/2507.03219

- V. Kumar, H. Sharma and S. Sood, "Economic Analysis of Tomato Cultivation in Himachal Pradesh," Indian J. Econ. Dev., vol. 17, no. 2, pp. 473–478, Jun. 2021, doi: 10.35716/IJED/21035. [Online]. Available: https://pubmed.ncbi.nlm.nih.gov/40464016/

- P. Patil, N. Yaligar and S. M. Meena, "Comparision of Performance of Classifiers - SVM, RF and ANN in Potato Blight Disease Detection Using Leaf Images," 2017 IEEE International Conferenceon Computational Intelligence and Computing Research (ICCIC), Coimbatore, India, 2017, pp. 1-5, doi: 10.1109/ICCIC.2017.8524301

- R. Yakkundimath, G. Saunshi, B. Anami and S. Palaiah, “Classification of rice diseases using convolutional neural network models,” Journal of The Institution of Engineers (India): Series B, vol.103, no. 4, pp. 1047– 1059, 2022.

- H. Yin, Y. H. Gu, C.-J. Park, J.-H. Park and S. J. Yoo, “Transfer learning-based search model for hot pepper diseases and pests,” Agriculture, vol. 10, no. 10, p. 439, 2020.

- Y. Kurmi, P. Saxena, B. S. Kirar, S. Gangwar, V. Chaurasia and A. Goel, “Deep cnn model for crops’ diseases detection using leaf images,” Multidimensional Systems and Signal Processing, vol. 33, no. 3, pp. 981–1000, 2022.

- S. S. Gaikwad, S. S. Rumma and M. Hangarge, “Fungi affected fruit leaf disease classification using deep CNN architecture,” International Journal of Information Technology, vol. 14, no. 7, pp. 3815–3824, 2022.

- M. Prabu and B. J. Chelliah, “Mango leaf disease identification and classification using a cnn architecture optimized by crossover-based levy flight distribution algorithm,” Neural Computing and Applications, vol. 34, no. 9, pp. 7311–7324, 2022

- K. Joshi, V. Kumar, S. Kumar and S. S. Gill, "A deep learning mobile application for tomato leaf nutrition deficiency identification with YOLOv8 and enhanced augmentation," Plant Science, vol. 364, Art. no. 112973, Jan. 2026, doi: 10.1016/j.plantsci.2025.112973