From Static Clusters to Adaptive Intelligence: A Learning Driven Clustering Paradigm for Agri-IoT

Shubham Kumar
Department of Electronics and Communication
National Institute of Technology Patna, India
shubhamk.pg24.ec@nitp.ac.in

Bharat Gupta
Department of Electronics and Communication
National Institute of Technology Patna, India
bharat@nitp.ac.in

Rakesh Ranjan
Department of Electronics and Communication
National Institute of Technology Patna, India
rr@nitp.ac.in

Abhilekh Prakash
Department of Electronics and Communication
National Institute of Technology Patna, India
abhilekhp.phd24.ec@nitp.ac.in

Abstract—As global food demands rise, the integration of Agri-IoT and Wireless Sensor Networks (WSNs) has become essential for transitioning toward data-driven agriculture through the continuous surveillance of soil and atmospheric conditions. However, these networks are severely constrained by limited battery resources, leading to uneven energy depletion, poor cluster formation and reduced network lifetime. Traditional clustering protocols such as LEACH, DEEC, PSO, ACO and FCM fail to ensure sustained sensing coverage and balanced energy utilization, especially in large scale deployments, resulting in increased communication overhead and degraded performance. This paper proposes an energy optimized hybrid framework for WSN based Agri-IoT systems. The approach integrates the DEEC protocol to identify high energy CH candidates, ensuring energy aware cluster head selection. An EEKA is employed to maintain spatial balance among CHs, while K-Means clustering forms compact clusters to minimize intra cluster communication cost. Furthermore, a Q-learning agent is incorporated to dynamically adapt network operations by monitoring residual energy, cluster load and packet delivery ratio. Based on these observations, the agent performs actions which are CH switching, node reassignment and transmission power control to sustain optimal network performance. The framework is implemented in both centralized and federated architectures to evaluate scalability and adaptability. Simulation results demonstrate that proposed method significantly outperforms existing methods. It is the only approach achieving 100% sector survivability (9/9 sectors alive) after 1000 rounds. The proposed method improves network lifetime by 57% over LEACH/DEEC and 45% over PSO/ACO/FCM, while reducing energy consumption by 33% and 16%, respectively. Furthermore, it achieves up to 240% higher throughput compared to LEACH/DEEC and 50% improvement over PSO/ACO/FCM, while maintaining high PDR. Among the two variants, the federated deployment demonstrates superior robustness, sustaining 3 times more surviving sectors, achieving 8% longer network lifetime and providing improved overall stability compared to the centralized approach.

Keywords—Wireless Sensor Networks (WSNs), Reinforcement Learning (RL), Q-learning, Distributed energy efficient clustering (DEEC), Energy Optimization, Internet of Things (IoT), K-Means Clustering, Low Energy Adaptive Clustering Hierarchy (LEACH), Particle Swarm Optimization (PSO), Ant Colony Optimization (ACO), Energy Efficient Knapsack Algorithm (EEKA)

- INTRODUCTION

In the current era of agriculture, the traditional, labor-intensive farming model is being replaced by data driven, automated systems to meet the global demand for food. The Agri-IoT serves as the overarching framework that connects physical objects to the internet, while WSNs act as the foundational "eyes and ears" on the ground, in which sensor nodes are spatially dispersed and make up these networks, which helps to monitor the environment and send the gathered data to a central BS or sink node for additional processing and analysis. This structure provides a powerful, real-time window into remote plantation locations without human involvement. Despite their great potential, WSNs encounter significant limitations in terms of network lifetime, reliability of communication and energy efficiency. The sensor nodes in WSNs have small, non-rechargeable batteries. Generally these nodes are deployed in remote locations and densely deployed which make accessing individual nodes to perform maintenance or replace batteries very difficult [3].  As a result, the battery depletes energy in nodes leads to network partitions, energy hole, potentially loses packets and decreases the overall performance of the network.  This scenario is particularly prevalent in applications which require long term continuous monitoring, such as precision agriculture, disaster prediction and health diagnostics, in which data has to flow reliably and continuous operation is a necessity.

Figure 1: Spatial deployment of IoT enabled wireless sensor nodes

Figure 1 illustrates a smart agriculture field with spatially distributed IoT sensor nodes organized into clusters for efficient data collection and communication with a central base station. To support such deployments, numerous routing and clustering protocols have been developed with a focus on energy optimization and efficiency. These approaches aim to balance power consumption among sensor nodes to prolong network lifetime and ensure reliable operation. Among these protocols, clustering based routing has shown one of the most effective means for energy efficiency [4]. The renowned clustering protocols are LEACH, Stable Election Protocol (SEP) and DEEC [3]–[5]. These protocols are mainly designed to minimize communication overhead and ensure a balanced distribution of energy consumption throughout the network. DEEC, in particular, selects CHs based on residual energy and average network energy, thereby extending the network lifetime more effectively than random or static CH selection methods such as LEACH. Ultimately, while SEP and DEEC significantly improve upon traditional methods by addressing node heterogeneity, they represent just the first step toward truly autonomous energy balance in the WSNs.

However, traditional clustering protocols still have limitations in adaptability, scalability and dynamic optimization. Most of the existing schemes assume homogeneous or quasi-static environments, where node energy dissipation, communication distance and network topology remain relatively stable. In real world IoT deployments, the energy dynamics, node heterogeneity and communication interference can vary dramatically over time. As a result, static or semi-static CH selection often leads to sub optimal cluster configurations, premature energy depletion of certain nodes and overall performance degradation [5]. Also, existing methods rarely incorporate adaptive learning or feedback mechanisms that can respond to constant changing environmental conditions or operational uncertainties. Thus the design of an intelligent and adaptive clustering mechanism becomes crucial for achieving long term sustainability [6]. So to mitigate the above mentioned limitations, this study introduces a Reinforcement Learning Driven Hybrid Clustering (RLHC) approach that fuses intelligent optimization techniques across multiple layers of the clustering process.

- Figure 2. Workflow of the proposed framework (RLHC)

Figure 2 represent the workflow of the  proposed framework which integrates energy normalized DEEC (Layer 1), centrality and spatiality focused cost function based EEKA (Layer 2), CH guided K-Means clustering (Layer 3), Q-Learning RL (Layer 4) a lightweight adaptive and energy aware architecture and finally, composite channel modeled multi-hop communication management (Layer 5).

The top layer DEEC is energy normalized and serves as the foundational layer, prioritizing nodes with higher residual energy for CH selection, EEKA filters and ranks CH candidates using a cost function that determines the optimal number of CHs, enabling the selection of the most suitable CHs to improve spatial balance and reduce inter-cluster communication overhead then the K-Means algorithm refines the cluster formation by minimizing intra cluster distances and balance the spatial distribution of nodes. The RL layer introduces a self-learning capability that dynamically adjusts CH selection, transmission parameters and cluster reconfiguration based on real time feedback such as residual energy, cluster load and PDR. The final layer characterizes radio energy consumption during data transmission and reception among sensor nodes, CHs and the base station. It adopts a composite channel model that integrates Rician fading for small-scale variations and log-normal shadowing for large-scale propagation effects to estimate realistic channel link gains [8]. Additionally, multi-hop routing using Dijkstra’s algorithm is employed to determine optimal data transmission paths from sensor nodes to the sink.

The RLHC framework accomplishes adaptive energy balancing, increased throughput, improved PDR and extended network lifetime by combining these five layers.  It represent the potential to close the gap between contemporary ML integrated adaptive optimization techniques and conventional deterministic routing methods.  Also opens the door for more dependable and sustainable sensor network deployments in real world settings by offering a solid, scalable and self-evolving solution for the future of IoT enabled WSNs.

Our contributions can be summarized as follows:

- A hybrid RL-driven clustering framework with centralized and federated architectures achieving 100% network survivability (after 1000 rounds) and real time adaptive optimization.

- Achieves up to 57% improvement in network lifetime, 33% reduction in energy consumption and 240% increase in throughput, with federated deployment as compared to the renowned optimization based algorithms (ACO/PSO/FCM), enhancing robustness and scalability.

The remaining part of this paper is organized as follows:

Section II represents a comprehensive literature review of existing clustering protocols and RL approaches in WSNs. Section III describe the theoretical and technical background related to energy efficient communication and clustering mechanisms. Section IV focus on the methodology of proposed RLHC framework in details. Section V presents the simulation setup, performance evaluation parameters and comparative analysis of the proposed RLHC protocol with existing approaches. Finally, Section VI provides the conclusion and highlights possible future research direction.

- RELATED WORKS

- WSNs have become an integral component of modern IoT systems, which enables various applications in domain such as healthcare, agriculture etc. However, they still face challenge regarding energy efficiency. Since sensor nodes are battery powered, the overall network performance heavily relies on the effective management and utilization of this energy. Over the years, researchers have proposed a variety of clustering protocols, optimization algorithms and adaptive learning strategies to address this challenge. This section reviews existing work in this field [9]. Clustering is well recognized as a foundational approach for energy efficient WSNs. The LEACH protocol is the most seminal contributions, introducing randomized rotation of CHs to evenly distribute energy consumption among nodes. It reduces the number of direct transmissions to the BS and organizes nodes into energy efficient clusters [10]. But still it has several limitations, including uneven CH distribution, lack of consideration for residual energy and poor performance in heterogeneous or large scale networks. These limitations motivated the development of variants that incorporate energy awareness and residual energy into CH selection [9].

- DEEC protocol is also one of the renowned protocol for achieving  energy efficiency in WSNs, which take residual energy in account to find the suitable CH which balance the energy of network and improve the network lifetime [11]. Few protocol has been developed by integrating RL such as LEACH RLC and ReLeC (enhance LEACH) for adaptive CH selection. These methods learn network states and make dynamic decisions about CH election to improve energy efficiency and extend the network lifetime. While the results are encouraging, RL based variants can add computational and communication overhead. The practical applicability of RL models such as ReLeC may be limited in large scale or highly dynamic WSNs due to convergence issues [12].  It is evident that RL based clustering protocols have demonstrated significant gains in throughput, network stability and energy efficiency.  However, these methods have certain drawbacks. For IoT nodes, the learning stage of RL models frequently adds computational and communication overhead. It may depletes the sensor node's energy, which is already scarce in terms of processing power.

- In order to address the limitations of conventional homogeneous protocols, researchers have developed methods that acknowledge a heterogeneous community of nodes. SEP (Stable Election Protocol) and DEEC are the two most well-known protocols that take into account the fact that nodes have different initial energy states and CH selection is based on residual energy states as well as energy distribution throughout the network. This ensures that nodes that are more energetic are more likely to accept the responsibilities of being a CH [13]. DEEC, in particular, increases functionality and network lifetime by extending LEACH to include energy heterogeneity and energy based CH selection methods. Other enhancements to the CH selection process such as EEKA reads the CH role allocation undergoing based on both node centrality and spatial distribution to promote more uniform CH allocation to really achieve reduced distances in intra cluster communication and energy consumption. Several metaheuristic algorithms, such as Particle Swarm Optimisation (PSO), Genetic Algorithms (GA) and Ant Colony Optimisation (ACO) have been extensively investigated in WSNs to support CH selection and network performance. Based on simulating the behavior of a swarm of like particles, PSO associates the best CH locations and reduces intra cluster distances [14]. Conversely, ACO uses a pheromone based path selection process to increase routing effectiveness and cluster formation. GA evolve the population of CH candidates through time, improving load balancing and energy efficiency. However, once an algorithm generates its optimal solution, like a GA, it typically does so offline, while other metaheuristic approaches often rely on iterative based processes [15]. These techniques are less effective in networks that are large, mobile, have changing topologies, or quickly fluctuating energy states.

- Clustering based on fuzzy logic provides a multi criteria decision making technique for CH selection, considering factors like residual energy, nodes density, distance to the cluster center and network traffic. MRCH (Modified RCH-LEACH) and others use fuzzy rule sets to choose eligible CH candidates. The method promotes a higher packet delivery ratio, provides improved network stability and increases overall energy efficiency. This approach contributes to clusters exhibiting dynamically adaptive characteristics in response to dynamic changes in the network by providing a flexible method for managing uncertainty in sensor networks [16]. However, some low power sensor nodes may restrict the use of fuzzy based computations due to their computations complexity.

- RL has become a powerful technique for control of dynamic and adaptive WSNs. An environment is developed using RL based notation of the network. Nodes or CHs are agents learning the optimal action to maximize long term utility, such as lifetime and energy efficiency. Q-learning, SARSA and DQN have been implemented to improve CH selection, cluster re-organisation and routing. Techniques such as EER-RL increase energy efficiency and lifetime by modifying CH roles in response to learnt energy patterns [17]. Similarly, Q-learning integrated with LEACH increase the protocol’s adaptability to variations in network topology. While these RL based approaches work well, RL often requires global knowledge, involves training in a centralized scenario, which may limit the scalability of RL based techniques for large scale dynamic deployments of WSNs. Hybrid clustering methods aim to combine the strengths of classical, meta heuristic, fuzzy and RL approaches. These methods address multiple challenges simultaneously. This covers energy balancing, spatial uniformity, CH optimisation and adaptability.  In order to maximise network efficiency, we discovered that multi layered hybrid models frequently incorporate energy aware CH selection, K-means or fuzzy based spatial clustering and RL based adaptive decision making.  The effectiveness of such hybrid frameworks has been demonstrated in recent work.  The ideal number of cluster and grid heads to balance energy consumption is determined by protocols such as EOCGS [18].  According to these studies, intelligent hybrid techniques can perform better than conventional methods in terms of network lifetime, energy balance and adaptability.

- Even with notable improvements in WSN clustering, existing methods continue to face several challenges. Metaheuristic approaches often needs heavy computation.  Classical methods frequently fail to prevent early node depletion, resulting in network partitioning. In contrast, Q-Learning introduces minimal computational overhead and enables the system to adapt effectively to dynamic environments. Moreover, most existing solutions rely on static or pre defined strategies, which are not enough to handle dynamic topologies or sudden energy fluctuations. So there is need of an adaptive lightweight framework which can handle all these edge case and outperform the existing algorithms.

- TABLE 1. Summary of energy efficiency studies in WSN

| - Sl. No. | - Paper & Year | - Technique Used | - Limitations (Research gap) |
| --- | --- | --- | --- |
| - 1 | - Farahzadi et al. “An Improved Cluster Formation Process in Wireless Sensor Networks to Decrease Energy Consumption” (2021) [19] | - BS selects CH based on node’s residual energy and proximity within predefined zones | - Assumes ideal energy estimation and homogeneous energy levels. BS assumes global knowledge of all node’s energy |
| - 2 | - Panchal et al. “EEHCHR: Energy Efficient Hybrid Clustering and Hierarchical Routing for Wireless Sensor Networks” (2021) [20] | - Hybrid clustering with hierarchical routing using FCM | - Increased routing complexity. Static clustering radius and considered homogeneous WSN scenarios. |
| - 3 | - Al-Kaseem et al. “Optimized Energy-Efficient Path Planning with Multiple Mobile Sinks” (2021) [21] | - Stable Election Algorithm (SEA) and residual energy for CH selection | - Scalability issues beyond 100 nodes. Simulation-only, no deployment |
| - 4 | - Prajapati et al. “Performance Analysis of LEACH with Deep Learning in Wireless Sensor Networks” (2022) [22] | - CNN-based CH selection using LEACH algorithm | - High computational overhead. Limited scalability. |
| - 5 | - Mohapatra et al. “Mobility Induced Multi-Hop LEACH Protocol in Heterogeneous Mobile Network” (2022) [23] | - Introduces a multi-hop extension of LEACH for heterogeneous mobile sensor networks | - Assumes uniform mobility. Limited scalability and fails in highly dynamic or large scale network conditions. |
| - 6 | - Gamal et al. “Enhancing Lifetime of WSNs Using Fuzzy Logic LEACH and PSO” (2022) [24] | - Hybrid clustering algorithm K-Mean and PSO for cluster formation | - Assumes static deployment and homogeneous node capabilities, limiting its applicability in highly dynamic or heterogeneous WSN |
| - 7 | - Bhatia et al. “Cluster Based Energy Efficient Routing Protocol using SA-LEACH to Wireless Sensor Networks” (2023) [25] | - LEACH and a Simulated Annealing (SA) meta-heuristic to pick a better (neighboring) node if available. | - High computational cost. Considers static and homogeneous networks, limiting adaptability |
| - 8 | - Abose et al. “Improving Wireless Sensor Network Lifespan with Optimized Energy-Conscious Routing” (2024) [26] | - Optimized energy conscious routing with a corner positioned base station | - Assumes ideal energy estimation. Does not consider node location or distance factor |
| - 9 | - El Khediri et al. “Energy-Efficient Cluster Routing Protocol for Wireless Sensor Networks” (2024) [27] | - Cluster based routing using hybrid meta heuristic algorithms | - Designed for homogeneous, static node deployments. Limited adaptability |
| - 10 | - Zhu et al. “Improved Soft-k-Means Clustering Algorithm for Balancing Energy Consumption in Wireless Sensor Networks” (2024) [28] | - Soft-k-means clustering with multi CH | - High computational cost. Limited adaptability |
| - 11 | - Tabatabaei et al. “New Energy Efficient Management Approach for Wireless Sensor Networks” (2025) [29] | - Hierarchical clustering approach with meta heuristic algorithms | - Assumes ideal energy estimation. Does not consider node location or distance factors. |
| - 12 | - Reddy et al. "AD-DDEEC Augmented Density-Based Distributed Energy-Efficient Clustering" (2025) [30] | - Implements a composite fitness function that dynamically adjusts sleep/wake cycles based on the distance to the Base Station. | - Fails to account for security-related energy drain; ignores the overhead of frequent control message exchanges in large-scale networks. |
| - 13 | - Sharma & Kumar "EEBMFO:Energy-Efficient Bat-Moth Flame Optimization for Routing" (2025) [31] | - Hybridizes Bat (echolocation) and Moth Flame (spiral movement) algorithms to find the shortest, most energy-dense paths | - Tendency to get trapped in local optima in irregular network topologies; ignores the "energy hole" problem near the sink. |
| - 14 | - Olayemi et al. "MLSRP Machine Learning-based Secure Routing Protocol" (2025) [32] | - Integrates light-weight Reinforcement Learning to predict node energy depletion and bypass failing nodes. | - The initial training phase consumes significant battery power before the network becomes efficient. |
| - 15 | - Veluswamy, "Trust-aware secure energy-efficient routing protocol using two-tier optimization" (2026) [33] | - Two-Tier Optimization: Combines trust management with energy metrics to select secure and efficient Cluster Heads (CH) | - Overhead for Trust Calculation, Calculating trust scores for every node increases the computational load and energy drain on low-end sensors. |
| - 16 | - A Comprehensive Survey of Routing Protocols for Underwater WSN" (2026) [34] | - Cross-layer Survey: Analyzes routing protocols specifically for the high-pressure, energy-scarce underwater environment. | - Specific environment focus: Protocols optimized for underwater acoustic communication are often inefficient for terrestrial radio-based WSNs. |
| - 17 | - Hartono et al. "Metaheuristic Approaches for Energy Optimization in WSN, A Systematic Review" (2026) [35] | - Metaheuristic Framework: A synthesis of hybrid algorithms (Machine Learning, Edge Computing, and UAV-assisted routing). | - Lack of real-world validation, 61% of current research remains purely simulation-driven, with few practical deployments in Industrial IoT. |
| - 18 | - Gawade et al. "I-HEED Intelligent Hybrid Energy-Efficient Distributed Clustering" (2026) [36] | - Uses a modified HEED algorithm with a fitness function based on node density and iterative competition for CH selection | - High computational overhead for low-power sensors; performance in high-mobility scenarios (like FANETs) remains untested. |
| - 19 | - Energy optimized scheduling in WSN using RL-HAPSO" (Nature, 2026) [37] | - RL-HAPSO: Combines Ant Colony (ACO) and Particle Swarm Optimization (PSO) with Q-learning Reinforcement Learning for adaptive scheduling | - High algorithmic complexity; requires significant computational resources to execute real-time reinforcement learning on constrained nodes |
| - 20 | - Sowbarnika, "Trust aware secure energy-efficient routing protocol for clustered WSNs" (2026) [38] | - Trust-Energy Integration, Uses a multi-factor fitness function to prioritize nodes with both high residual energy and high reliability | - Latency in Large Networks, The two-tier optimization process can introduce delays in route discovery for large-scale deployments. |
| - 21 | - "Enhanced Secretary Bird Optimization for Energy-Efficient Cluster Head Selection" (2026)[39] | - Secretary Bird Optimization (SBOA) A bio-inspired metaheuristic that mimics the hunting behavior of birds to find optimal CH locations | - Stalling in Local Optima: Can become inefficient in complex or irregular network topologies where the "prey" (optimal node) is harder to isolate |
| - 22 | - Intelligent Energy-Aware Routing via Protozoa Behavior" (2026)[40] | - Protozoa Behavior Modeling: Mimics the adaptive movement and resource consumption of simple organisms to determine data paths. | - Limited Stability: The biological analogy may not hold up during massive network surges or sudden node failures (partitioning) |

- THE METHODOLOGY

This paper presents a Reinforcement Learning Driven Hybrid Clustering (RLHC) framework. It improve energy efficiency and operational lifetime in WSNs. The whole framework divided into a five-layer architecture, in which each layer contribute its own advantage. The whole framework collectively optimizes CH roles and network management through real time feedback. From simulation results we can confirm that the integrated RLHC framework delivers improved throughput, higher packet delivery ratio (PDR) and extends network lifetime. It merges the deterministic optimization with machine learning guided adaptation. This is making whole system robust across diverse IoT enabled WSN scenarios. The RLHC framework composed of five integrated layers designed to improve energy efficiency and clustering in WSNs.

Figure 3. Layer-wise representation of proposed framework (RLHC)

The Figure 3, illustrates the proposed multi-layer architecture for energy-efficient clustering and communication in wireless sensor networks. Initially, high energy nodes are prioritized as potential cluster head candidates, followed by centrality and spatial filtering to ensure optimal distribution. Compact clusters are then formed using a CH-guided K-means approach. An adaptive Q-learning mechanism further refines cluster head selection and transmission parameters. Finally, a composite channel model enables efficient multi-hop communication between cluster heads and the base station.

Layer 1: Energy Normalized Distributed Energy Efficient Clustering Protocol

DEEC acts as the fundamental layer of the proposed RLHC framework. In this work, it is enhanced into an Energy-Normalized DEECP (EN-DEECP), where the conventional energy-aware clustering mechanism is refined through normalization of node energies to improve fairness, stability and comparability. DEECP inherently operates in a heterogeneous network setting, consisting of a combination of, Normal nodes (baseline energy) and Advanced nodes (higher initial energy). This heterogeneity allows advanced nodes to take on more energy-intensive roles such as CH, thereby improving network lifetime. In the proposed EN-DEECP enhancement, an additional energy normalization constraint is introduced. Specifically, the total initial energy of the network is kept constant across all comparative algorithms, regardless of node heterogeneity. This ensures that the sum of energy across all nodes remains equal for every protocol under evaluation. To promote performance improvements are due to algorithmic efficiency, not excess energy availability. Furthermore, CH selection is based on normalized residual energy, meaning each node’s energy is evaluated relative to the current network energy state rather than absolute values. This prevents dominance of high-energy nodes and promotes balanced energy dissipation. In heterogeneous IoT-enabled WSN environments, this enhanced formulation not only optimizes initial CH selection but also provides a normalized and stable foundation for higher-layer processes such as spatial filtering, clustering, and Q-learning-based adaptation, ultimately leading to improved load balancing and energy efficiency.

Etotalnormalized=N⋅E0                                                  (1)

Here Equation (1) represents the total normalized energy of the network, where N denotes the total number of nodes and E0 represents the initial energy of each node.

Enormal'=E01+mα                                                   (2)

Eadvanced'=E0(1+α)1+mα                                             (3)

Equation (2) represents the initial energy of normal nodes, where E0 denotes the energy assigned to each normal node, and equation (3) represents the initial energy of advanced nodes, where α is the energy factor indicating the additional energy of advanced nodes compared to normal nodes.

N(1−m)Enormal'+NmEadvanced'=NE0              (4)

Pi=popt⋅Ei(r)Eavg(r)                                                        (5)

Equation (4) verifies the total energy consistency after normalization, ensuring that the sum of energies of all nodes equals N⋅E0, where m denotes the fraction of advanced nodes and (1- m) represents the fraction of normal nodes, Equation (5) represents the probability of node i becoming a cluster head, where Popt is the optimal cluster head probability, Ei(r) is the residual energy of node i at round r, and Eavg(r) is the average residual energy of the network.

T(n)=Pi1−Pir mod 1Piifn∈G0otherwise                     (6)

Equation (6) represents the threshold function used for CH selectinn, where T(n) is the threshold for node n, and G is the set of nodes that have not been selected as cluster heads in the last, N represents total numbers of nodes, R represents total numbers of rounds.

Layer 2: EEKA with Centrality and Spatially Aware Cost Function Optimization

- Following the DEECP layer, the EEKA operates as the second optimization layer within the RLHC framework.  EEKA refines this process by optimizing the spatial distribution and number of CHs using a constrained knapsack formulation. In this layer, each potential CH candidate identified by DEECP is evaluated as an item in the knapsack problem, where parameters such as node centrality, residual energy and communication distance to the BS act as utility factors. It focus on maximizing the overall CH efficiency by considering minimum use of total energy consumption. The selection of the CH set is spatially balanced and energy efficient [33]. It guarantees uniformly coverage of the sensing field and limits overlong intra cluster communication distances. This selection eliminates energy hotspots and promotes uniform energy dissipation across the entire network's sensing field. The EEKA approach also resolves areas of spatial irregularity typically common in DEECP or LEACH based clustering approaches. Furthermore, the optimal CH arrangement developed within EEKA provides a better input to the K-Means clustering layer, which refines the cluster boundaries to ensure even less intra cluster distances and transmission costs.

Figure 4: Internal block diagram of the EEKA algorithm

The internal design of EEKA is depicted in Figure 4. It works by obtaining candidate CH, node information about energy and position. It then calculates average inter node distances and determines node centrality using a combined utility function (Equation 7) to rank the candidates. The top K candidates are selected as CH to achieve the most balanced energy consumption and an even cluster distribution throughout the network.

Ui=αNormEiE2+βNorm1di+ε                (7)−γNorm(Hi)

Here the utility function Ui represents the utility score (or fitness value) for node. A higher value typically makes a node more likely to be selected for a specific task (like routing or being a leader) and spatial centrality into a single quantitative metric. Hi typically represents the Hop Count (number of transmission steps to the sink) or Node Degree.

Layer 3:  CH-guided K-Means Algorithm

In the third layer of the RLHC framework, K-Mean is used as a clustering algorithm. It refine the cluster formation process based on the CHs selected by the EEKA layer. While the first and second layers together determine the most energy efficient and spatially balanced CHs, the K-Means algorithm ensures that the remaining sensor nodes are optimally associated with these CHs to minimize intra cluster communication cost.

Figure 5: Workflow of CH-guided K-Mean algorithm

Unlike conventional K-Means, where centroids are initialized randomly, the proposed approach utilizes the pre selected CHs as initial centroid points as represented in figure (5). This CH-guided initialization significantly improves clustering performance by accelerating convergence, reducing sensitivity to random initialization and ensuring that centroids are already positioned in energy efficient and spatially optimal locations. As a result, the clusters formed are more balanced and aligned with the network topology, leading to reduced communication overhead and improved energy efficiency. The process involves three main steps: centroid initialization, mean (centroid) update and convergence check.

(8)

The objective function of K-Mean clustering process is given in Equation 8, where Ci represents the set of nodes in cluster i, μi is the centroid of cluster i and sq(x−μi) denotes the squared Euclidean distance between node x and its cluster centroid. This algorithm partitions a set of N sensor nodes into K clusters by iteratively minimizing the sum of squared Euclidean distances between each node and its assigned CH.

Layer 4:  Reinforcement Learning (RL)

Layer 4 incorporates a RL agent to provide self-learning, adaptive control for the WSN. By utilizing real-time feedback from the network itself, it optimally adjusts and changes the CH selection, transmission parameters and cluster configurations dynamically. The RL agent uses the Q-Learning algorithm to obtain improved, optimal CH selection and transmission decisions/parameters. The learning agent focuses on maximizing long-term objectives, such as network lifetime and energy efficiency, rather than optimizing short term goals [34]. The RL agent is able to continuously improve and update its strategies over time and avoid some of the limitations associated with static, rule based systems. The RL agent also works in conjunction with the previous layers to improve the overall decision making process, while maintaining all capabilities for scalable, efficient and adaptive clustering. Although the RL agent does involve a small amount of computational overhead but it is crucial for long term dynamic system.

Figure 7. RL based Decision-Making Framework

In the diagram above shows that, with RL the system is able to flexibly adjust to new conditions and continually improve its decisions in a more optimal way as time stoops. This is what makes it very applicable to real-time IoT applications.

- In Layer 4, a Q-learning agent is implemented with the sole objective of observing the current network state, taking actions (e.g., CH re-assignment, switching CHs and changing transmission power) and receiving rewards as experience. After executing an action, it receives feedback from the environment, including updated node energies, cluster configurations and network metrics (e.g., PDR, throughput) and computes a reward. This experience is then used to update the Q-table, refining future decision making. By introducing adaptive learning, this layer transforms static cluster management into a self-learning mechanism, enabling the network to optimize long term objectives such as energy efficiency and network lifetime. This layer is very crucial and enables the system to adapt effectively to dynamic changing environments.

- Equation 14 is used to update the Q-table, where α is the learning rate that determines how much new information overrides old knowledge, γ is the discount factor that defines the importance of future rewards and ‘a’ represents the action taken by the agent. This update allows the agent to iteratively improve its policy by balancing immediate rewards with long-term benefits, enabling more optimal decision making over time.

Figure 8. Q-Learning agenrt decision making framework

- Figure 8 illustrates how the agent interacts with the environment to make adaptive chnages and fine tune various parameters.

- Layer 5: Communication Layer

- Layer 5 is responsible for wireless communication and energy modeling during data transmission between nodes, CHs and the BS. This layer simulates the radio energy dissipation model, which estimates the transmission and reception energy costs based on the distance between nodes and the amount of data transmitted. In each communication round, member nodes transmit sensed data to their respective CHs, which then perform data aggregation and forward the aggregated packet to the BS using a single-hop communication mechanism [35]. In the proposed model, wireless communication is simulated using the first-order radio energy model that is Additive White Gaussian Noise (AWGN) channel., which incorporates both free-space (d²) and multipath (d⁴) channel models. For short-range intra cluster communication (d<d0), the free-space model is used, assuming line-of-sight propagation. For long-range transmissions (d≥d0), the multipath model is applied to account for non-line-of-sight energy loss. This dual channel approach effectively captures large-scale path loss while maintaining computational simplicity, providing a realistic approximation of WSN communication behavior.

- Equation 15, computes the energy needed to transmit a k-bit packet over a distance d, Longer distances (d≥ d0) incur significantly higher energy costs due to multi path fading, where k is size of the data packet in bits, E_elec is energy required per bit for transmission/reception circuitry, ε_fs is amplifier energy for free-space model, ε_mp is amplifier energy for multipath model, d is distance between sender and receiver, d0 is threshold distance to switch between free-space and multi path.

- Equation 16, calculates the energy required to receive k bits, receiving costs are independent of distance, as no amplification is needed. Equation 17, E_DA is energy required for data aggregation for k bits before transmission.

Equation 18 and 19 calculates the total energy consume by the CHs and CMs, when considering no multi path fading.

The proposed architecture intends to raise the energy efficiency and reliability of communication for WSNs in Agri-IoT contexts. Features the multi-layer architecture that effectively blends conventional clustering approaches and smart reinforcement learning for optimization. In this system, sensor nodes deployed randomly throughout the agricultural field. The sensor nodes will continuously collect environmental data such as soil moisture, humidity and temperature. The data collected from the sensor nodes will then be sent to a base station, where the data will be processed and decisions made with the results. The architecture consists of five functional layers. In the first layer, the nodes are deployed and run DEECP, to identify the best options to serve as CH. In the second layer, the remaining energy and degree of centrality of the candidates are identified and ranked. In the third layer, the K-Means clustering algorithm is performed, linking the non CH nodes to the closest CH will reduce communication distances between nodes in the cluster and distribute energy consumption. The fourth layer introduces a Q-learning agent that intelligently optimizes CH selection based on the ever changing state of the network. The agent observes the environment, selects actions such as reassigning CHs or adjusting transmission power and updates its Q-table based on the obtained rewards. Ultimately, the fifth layer is focused on the communication, where member nodes transmit their data to CHs and aggregated packets will be forwarded to the BS. The radio communication model used in this work incorporates free-space (d²) and multipath (d⁴) channel propagation characteristics which represent conditions of agricultural fields where line-of-sight and non-line-of-sight transmissions are expected. Due to this holistic design, extends network lifetime and ensure data reliability.

- Figure 9. System architecture of the proposed algorithm

- Algorithm 1: RHLC - Reinforcement Learning Driven Hybrid Clustering

Initialize:

Network parameters, node positions and energy.

Q-Table Parameters

Set round counter r = 0.

While (any node is alive):

Layer 1 (DEEC): Estimate CH probability based on residual energy ratio.

Layer 2 (EEKA): Divide WSN field into spatial sectors and select energy-efficient CH candidates using node centrality and residual energy.

Layer 3 (K-Means): Form compact clusters by associating nodes with the nearest CH closest to the centroid.

Layer 4 (Q-Learning):   Observe state (residual energy, distance, alive ratio, pdr, cluster info) Then choose action using ε-greedy policy (CH reassignment / maintain). Then update Q-table using Bellman update equation.

Layer 5 (Communication): Transmit data: Member nodes to CH (Free-space, d²), CH to BS (Multipath, d⁴) under AWGN channel. Then update node energies and performance metrics (PDR, throughput etc).

Increment round counter r.

End While.

Table 2: WSN parameters for Agri-IoT WSNs

| Network Parameters | Value |
| --- | --- |
| WSN area size | 500m x 500m |
| Number of nodes | 200 |
| Initial energy | 2 Joules |
| Packet size | 512 bytes |
| Data aggregation energy | 5n J/bit/signal |
| Transmit energy | 50n J/bit |
| Receive energy | 50n J/bit |
| Free space loss | 10n J/bit |
| Multipath loss | 0.0013p J/bit |
| Simulation Rounds | 1000 |

The simulation will evaluate WSN performance by tracking

the number of dead nodes, network lifetime, energy consumption, throughput and improvement in PDR.

- RESULTS AND DISCUSSION

This section presents the performance evaluation results of the optimized algorithms built on top of LEACH and DEEC and the proposed mechanism. The analysis focuses on the network lifetime, energy consumption, throughput and packet delivery ratio. The simulations assume that all nodes are either stationary or exhibit only micro-mobility, energy losses due to dynamic random channel conditions are neglected.

Figure 10. Number of dead nodes during network operation

The relative performance of different protocols in terms of network lifetime is shown in Figure 10.  The first node dies around node 60 in the LEACH protocol, which exhibits the earliest network degradation.  By extending the first node death to approximately 100, DEEC provides a minor improvement.  Algorithms such as FCM and PSO improve the network's overall stability, ACO typically performs marginally better in most situations. In terms of protocol performance with respect to the first node failure time metric, RLHC, demonstrates that the first node failure occurs around 150 rounds, which is about 1.2 times later than ACO and 3.8 times later than LEACH. The reduced rate of nodes depleting energy suggest that RLHC's adaptive learning method provides a more equitable distribution of energy throughout the network.

Data throughput displays a similar trend depicted in Figure 11. LEACH shows the lowest total throughput at roughly 23,000 packets after 1000 rounds, primarily due to early node deaths. DEEC and Fuzzy C-Means bring throughput levels up to about 44,000 packets and 55,000 packets, due to their enhanced CH selection process. The same trend is continued further with PSO and ACO, which both maintain poor stability throughout the simulation, while also improving throughput, peaking at roughly 53,000 and 71,000 packets, respectively. The proposed RLHC protocol presents a dramatic reduction in throughput as the RLHC protocol and achieves the highest cumulative and final throughput at nearly 75,000 packets sent to the BS. This is a whopping +226% greater than LEACH and +5% more packets sent than ACO, thereby supporting the hypothesis that reinforcement learning maximizes the operation of the network while decreasing unnecessary communication losses.

Figure 11. Cumulative throughput during network operation

Figure 12. Total energy consumption during network operation

In Figure 12, we show the effectiveness of each protocol in utilizing the available energy resources. Here, we begin with the two protocols LEACH and DEEC, which consume energy quickly, showing total consumption of almost 400 J around the 300th round, leading to unanticipated failures of nodes. In contrast, the FCM and PSO protocols demonstrated slower and more controlled energy consumption towards the end of the simulation, with total consumption of around 370 J and 380 J. The ACO protocol showed even better energy use which around 365 J by round 1000. The protocol we proposed, RLHC consumed the least energy overall with consumption of around 360 J after the 1000th round which indicates a balanced energy utilization.

Figure 13. Total PDR during network operation

The comparative analysis of the Total Packet Delivery Ratio (PDR) provided in Figure 13 showcases the performance of different algorithms including LEACH, DEEC, Fuzzy C-Means, PSO, ACO and the new proposed RLHC method. Overall, the results indicate that all algorithms have a high PDR, which reflects that the capability of data delivery is reliable within the network. However, there is an increase in the proposed RLHC method which records the highest PDR of 99.88%, followed closely by ACO with 99.76%, PSO with 99.73%, Fuzzy C-Means with 99.71%, DEEC with 99.63% and lastly LEACH with the lowest recorded PDR of 99.51%. The improved PDR in the RLHC protocol signifies that it dynamically optimizes communication paths while minimizing packet loss caused by energy depletion or transmission failures.

Figure 14 compares the operational lifetime of the network for the same set of algorithms, measured in terms

- Figure 14. Operational lifetime of the network

of the total number of rounds completed before the network becomes non-functional. The proposed RLHC approach achieves  the  longest  operational  duration of  7128  rounds outperforming all other algorithms.

Table 3: Comparison of overall performance

| Method | LEACH | DEEC | FCM | PSO | ACO | RLHC |
| --- | --- | --- | --- | --- | --- | --- |
| Network Lifetime (rounds) | 1110 | 4008 | 3862 | 5904 | 3730 | 7128 |
| Energy Consumption (J) | 400 | 390 | 380 | 380 | 385 | 360 |
| Packet Delivery Ratio (%) | 99.51 | 99.63 | 99.73 | 99.71 | 99.76 | 99.88 |
| Throughput(bits) | 23000 | 44000 | 55000 | 53000 | 71000 | 75000 |
| First Node Dead (FND) | 8 | 12 | 4 | 9 | 14 | 21 |
| Half Node Dead (HND) | 83 | 81 | 103 | 257 | 120 | 145 |
| Last Node Dead (LND) | 1110 | 4008 | 3862 | 5904 | 3730 | 7128 |
| Energy Model | Homogeneous | Homogeneous | Homogeneous | Hetrogeneous | Hetrogeneous | Hetrogeneous |

As shown in Table 3, the RLHC protocol shows noticeable improvements over earlier clustering and optimization based routing techniques in nearly every important performance metric. RLHC achieves the highest network lifetime (7128 rounds), lowest energy consumption (360 J) superior packet delivery ratio (99.88%) and highest throughput (75000), which demonstrating its ability to maintain reliable communication and balanced energy utilization. Also, the delayed occurrence of the first and last node deaths highlights the protocol’s enhanced stability and robustness in heterogeneous environments. These results collectively validate the effectiveness of the RLHC  in WSNs.

- CONCLUSION

In this study, RLHC protocol was proposed to improve the energy efficiency and lifetime of heterogeneous WSNs. By integrating Q-learning with multi layered optimization mechanisms, RLHC dynamically adapts to network conditions and optimizes CH selection and data transmission paths. Simulation results clearly demonstrate that RLHC surpasses traditional protocols such as LEACH, DEEC as well as optimized protocol built on top of traditional algorithm such as FCM, PSO and ACO in terms of network longevity, throughput and energy conservation. The proposed approach effectively balances the energy load among nodes, reduces early node deaths and sustains overall network connectivity for extended periods.

Future work will focus on extending RLHC to mobile and large scale sensor networks, incorporating Simulated Annealing (SA) for further improvement and validating performance under real time deployment scenarios.

- REFERENCES

- S. M. Chowdhury and A. Hossain, ‘‘Different energy saving schemes in wireless sensor networks: A survey,’’ Wireless Pers. Commun., vol. 114, no. 3, pp. 2043–2062, Oct. 2020.

- F. M. Salman, A. A. Mohammedand A. F. Mutar, “Optimization of LEACH protocol for WSNs in terms of energy efficient and network lifetime,” Journal of Cyber Security and Mobility, vol. 12, no. 3, pp. 275–296, May 2023, doi: 10.13052/jcsm2245-1439.1232.

- G. M. T. Tamilselvan and K. Gandhimathi, ‘‘Network coding based energy efficient LEACH protocol for WSN,’’ J. Appl. Res. Technol., vol. 17, no. 1, pp. 251–267, Jun. 2019

- W. Heinzelman, A. Chandrakasanand H. Balakrishnan, “Energy- efficient communication protocol for wireless microsensor networks,” in Proc. 33rd Hawaii Int. Conf. System Sciences, 2000, pp. 1–10, doi: 10.1109/HICSS.2000.926982.

- A. Chehri, R. Saadane, N. Hakemand H. Chaibi, ‘‘Enhancing energy efficiency of wireless sensor network for mining industry applications,’’ Proc. Comput. Sci., vol. 176, pp. 261–270, Jan. 2020.

- P. Kathiroli and K. Selvadurai, ‘‘Energy efficient cluster head selection using improved sparrow search algorithm in wireless sensor networks,’’ J. King Saud Univ.-Comput. Inf. Sci., vol. 2021, pp. 1–12, Sep. 2021.

- R. S. Sutton and A. G. Barto, “Reinforcement learning: An introduction,” MIT Press, 2nd ed., 2018. [Online]. Available: http://incompleteideas.net/book/the-book-2nd.html

- H. Zhang, G. Liuand T. Jiang, “Parameter configuration scheme for optimal energy efficiency in LoRa-based wireless underground sensor networks,” IEEE Transactions on Vehicular Technology, vol. 74,  no.  6,  pp.  9961–9973,  Jun.  2025,  doi: 10.1109/TVT.2025.3351526.

- B. R. Al-Kaseem, Z. K. Taha, S. W. Abdulmajeedand H. S. Al- Raweshidy, “Optimized energy–efficient path planning strategy in WSN with multiple mobile sinks,” IEEE Access, vol. 9, pp. 79994– 80007, Jun. 2021, doi: 10.1109/ACCESS.2021.3087086.

- F. M. Salman, A. A. Mohammedand A. F. Mutar, “Optimization of LEACH protocol for WSNs in terms of energy efficiency and network lifetime,” Journal of Cyber Security and Mobility, vol. 12, no. 3, pp. 275–296, May 2023, doi: 10.13052/jcsm2245-1439.1232.

- M. Gamal, N. E. Mekky, H. H. Solimanand N. A. Hikal, “Enhancing the lifetime of wireless sensor networks using fuzzy logic LEACH technique-based particle swarm optimization,” IEEE Access, vol. 10, pp. 36935–36948, Mar. 2022, doi: 10.1109/ACCESS.2022.3163254.

- A. Panchal and R. K. Singh, “EOCGS: Energy efficient optimumnumber of cluster head and grid head selection in wireless sensor networks,” Telecommun. Syst., vol. 78, no. 1, pp. 1–13, Apr. 2021, doi:10.1007/s11235-021-00806-w.

- R. K. Singh, S. Verma, A. Panchal and S. Dubey, “Modified RCHLEACH (MRCH) for wireless sensor networks (WSN),” in Proc. 9th Int. Congr. Inf. Commun. Technol. (ICICT). Singapore: Springer, Jan. 2024, pp. 331–340, doi: 10.1007/978-981-97-3559 4_26.

- K. Yilmaz, R. Kara and F. Katircioglu, “Energy-efficient hybrid adaptive clustering for dynamic MANETs,” IEEE Access, vol. 13, pp. 51319–51331, 2025, doi: 10.1109/ACCESS.2025.3552232.

- S. Kaviarasan and R. Srinivasan, “A novel spider monkey optimized fuzzy C-means algorithm (SMOFCM) for energy-based cluster-head selection in WSNs,” Int. J. Electr. Electron. Res., vol. 11, no. 1, pp. 169–175, Mar. 2023, doi: 10.37391/ijeer.110124.

- S. K. Chaurasiya, S. Mondal, A. Biswas, A. Nayyar, M. A. Shah and R. Banerjee, “An energy-efficient hybrid clustering technique (EEHCT) for IoT-based multilevel heterogeneous wireless sensor networks,” IEEE Access, vol. 11, pp. 25941–25958, 2023, doi: 10.1109/ACCESS.2023.3254594.

- A. Aleem and R. Thumma, “Hybrid energy-efficient clustering with reinforcement learning for IoT-WSNs using knapsack and K- means,” IEEE Sensors Journal, vol. 25, no. 15, pp. 30047–30058, Aug. 2025, doi: 10.1109/JSEN.2025..

- R. Alsaqour, E. S. Ali, R. A. Mokhtar, R. A. Saeed, H. Alhumyaniand M. Abdelhaq, “Efficient energy mechanism in heterogeneous WSNs for underground mining monitoring applications,” IEEE Access, vol. 10, pp. 100123–100138, Jul. 2022, doi: 10.1109/ACCESS.2022.3188654.

- Farahzadi, H. R., Langarizadeh, M., Mirhosseini, M., & Fatemi Aghda, S. A. (2021). An improved cluster formation process in wireless sensor network to decrease energy consumption. Wireless Networks, 27(2), 1077–1087. https://doi.org/10.1007/s11276-020-02485-y.

- Panchal, A., & Singh, R. K. (2021). EEHCHR: Energy Efficient Hybrid Clustering and Hierarchical Routing for Wireless Sensor Networks. Ad Hoc Networks, 123, 102692. https://doi.org/10.1016/j.adhoc.2021.102692.

- Al-Kaseem, B. R., Taha, Z. K., Abdulmajeed, S. W., & Al-Raweshidy, H. S. “Optimized Energy-Efficient Path Planning Strategy in WSN with Multiple Mobile Sinks.” IEEE Access, vol. 9, 2021, pp. 82833-82847. https://doi.org/10.1109/ACCESS.2021.3087086

- Prajapati, H. K., & Joshi, R. (2022). Performance Analysis of LEACH with Deep Learning in Wireless Sensor Networks. International Journal of Electronics and Telecommunications, 68(4), 799–805. doi: 10.24425/ijet.2022.143888.

- Mohapatra, S., Behera, P. K., Sahoo, P. K., Bisoy, S. K., Hui, K. L., & Sain, M. (2022). Mobility Induced Multi-Hop LEACH Protocol in Heterogeneous Mobile Network. IEEE Access, 10, 132895–132907. https://doi.org/10.1109/ACCESS.2022.3228576.

- Gamal, M., Mekky, N. E., Soliman, H. H., & Hikal, N. A. (2022). Enhancing the Lifetime of Wireless Sensor Networks Using Fuzzy Logic LEACH Technique‑Based Particle Swarm Optimization. IEEE Access, 10, 36935‑36948. https://doi.org/10.1109/ACCESS.2022.3163254.

- Chekuri, S. & Bhandari, B. N. (2023). Cluster Based Energy Efficient Routing Protocol using SA‑LEACH to Wireless Sensor Networks. Indian Journal of Science and Technology, 16(7): 492‑500. DOI: 10.17485/IJST/v16i7.425.

- Abose, T. A., Tekulapally, V., Megersa, K. T., Kejela, D. C., Daka, S. T., & Jember, K. A. (2024). Improving wireless sensor network lifespan with optimized clustering probabilities, improved residual energy LEACH and energy efficient LEACH for corner‑positioned base stations. Heliyon, 10(14), e34382. https://doi.org/10.1016/j.heliyon.2024.e34382.

- El Khediri, S., Selmi, A., Khan, R. U., Moulahi, T., & Lorenz, P. (2024). Energy‑Efficient Cluster Routing Protocol for Wireless Sensor Networks Using Hybrid Metaheuristic Approaches. Ad Hoc Networks, 158, 103473. https://doi.org/10.1016/j.adhoc.2024.103473.

- Zhu, B., Bedeer, E., Nguyen, H. H., Barton, R., & Henry, J. (2024). Improved Soft‑k‑Means Clustering Algorithm for Balancing Energy Consumption in Wireless Sensor Networks. arXiv preprint arXiv:2403.15700.

- Tabatabaei, S. (2025). New Energy Efficient Management Approach for Wireless Sensor Networks in Target Tracking Using Vortex Search Algorithm. Heliyon, 11(5), e42867. https://doi.org/10.1016/j.heliyon.2025.e42867

- H. H. El-Sayed and Z. M. Hashem, “Comparison of the new versionof DEEC protocol to extend WSN lifetime,” EURASIP J. WirelessCommun. Netw., vol. 2023, no. 1, p. 56, Jul. 2023, doi: 10.1186/s13638-023-02265-0.

- B. Zeng, J. Deng, Y. Dong, X. Yang, L. Huang and Z. Xiao,“A whale swarm-based energy efficient routing algorithm for wireless sensor networks,” IEEE Sensors J., vol. 24, no. 12, pp. 19964–19981, Jun. 2024, doi: 10.1109/JSEN.2024.3390424.

- A. Verma, S. Kumar, P. R. Gautam, T. Rashid and A. Kumar, “Fuzzy logic based effective clustering of homogeneous wireless sensor networks for mobile sink,” IEEE Sensors J., vol. 20, no. 10, pp. 5615–5623, May 2020, doi: 10.1109/JSEN.2020.2969697.

- M. U. Younus, M. K. Khan and A. R. Bhatti, “Improving the softwaredefined wireless sensor networks routing performance using reinforcement learning,” IEEE Internet Things J., vol. 9, no. 5, pp. 3495–3508, Mar. 2022, doi: 10.1109/JIOT.2021.3102130.

- H. Dutta, A. K. Bhuyan and S. Biswas, “Contextual deep reinforcementlearning for flow and energy management in wireless sensor andIoT networks,” IEEE Trans. Green Commun. Netw., vol. 8, no. 3, pp. 1233–1244, Sep. 2024, doi: 10.1109/TGCN.2024.3358230.

- S. Mohapatra, P. K. Behera, P. K. Sahoo, S. K. Bisoy, K. L. Huiand M. Sain, “Mobility induced multi-hop LEACH protocol in heterogeneous mobile network,” *IEEE Access*, vol. 10, pp. 132895–132907, Dec. 2022, doi: 10.1109/ACCESS.2022.3228576.

- Q. Zhu, X. Wu, J. Wangand Z. Xu, “Deep reinforcement learning-based UAV trajectory planning for energy-efficient wireless sensor networks,” IEEE Transactions on Vehicular Technology, vol. 69,  no.  12,  pp.  15646–15655,  Dec.  2020,  doi: 10.1109/TVT.2020.3035872.